MANGALORE REFINERY AND PETROCHEMICALS LIMITED
TECHNICAL SERVICES & ASSET INTEGRITY DIVISION
CONFIDENTIAL / INTERNAL APPROVAL NOTE

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

| Equipment Tag: B-101-Crude-Distillation-Furnace-Coil | Classification: On-Premise Sovereign Verified |
| --- | --- |
| Security Compliance: 100% Air-Gapped | Department: Mangalore Refinery & Petrochemicals Limited |

1.0 SUBJECT
Approval for Continued Operation / Retubing Schedule of B-101-Crude-Distillation-Furnace-Coil under ASME B31.3 Standard

2.0 BACKGROUND & OPERATIONAL CONTEXT
Routine ultrasonic non-destructive testing (NDT) was conducted on B-101-Crude-Distillation-Furnace-Coil during the Q3 turnaround. The component operates at a design pressure of 4.0 MPa in the heavy crude distillation loop. Historical operational correspondence indicates an average localized corrosion rate of 0.25 mm/year.

3.0 NON-DESTRUCTIVE TESTING & INSPECTION FINDINGS
Ultrasonic thickness gauging at 16 test grid points revealed a minimum measured wall thickness of 7.5 mm. No structural pitting or circumferential cracking was observed on visual inspection.

4.0 ASME B31.3 CODE CALCULATIONS & REMAINING LIFE
As per ASME B31.3 Process Piping Code calculations verified in the on-premise execution sandbox: Minimum design thickness required = 6.2 mm (inclusive of 3.0 mm corrosion allowance). Calculated safe margin = 1.3 mm, yielding an estimated remaining safe operational life of 5.2 years. Engineering Integrity Assessment Status: APPROVED_SAFE.

5.0 FINANCIAL IMPLICATIONS & BUDGETARY PROVISION
No immediate emergency capital expenditure is required for replacement. Estimated budgetary provision of INR 45,00,000/- is recommended for routine scheduled coil retubing in the FY 2028-29 Annual Overhaul Plan.

6.0 DELEGATION OF POWER (DOP) REFERENCE
Item 4.2(b) of MRPL Delegation of Power (DOP) — Maintenance and Integrity Certification of Pressure Vessels.

7.0 RECOMMENDATION & PROPOSAL FOR APPROVAL
In view of the verified ASME B31.3 compliance (5.2 years safe life remaining), it is recommended to approve continued operation of B-101-Crude-Distillation-Furnace-Coil with next ultrasonic inspection scheduled after 24 months.

Submitted for Approval:

_______________________
Chief General Manager (Technical Services)
Mangalore Refinery & Petrochemicals Limited
Mangalore Refinery and Petrochemicals Limited